FlexiDon iX

Requirements & Project Consolidation

A multipurpose, configuration-defined industrial edge platform

Status: requirements-led — proof-of-concept implementation now underway (web-first)

Document version 0.3  ·  2026-06-28

# Table of Contents

# 1. Document Purpose

A single consolidated record of (a) the product requirements and guiding principles gathered so far, and (b) what has actually been built in the proof-of-concept. This is a living document — decisions are added as they are settled, and open questions are tracked explicitly rather than hidden.

Working status: the project remains requirements-led — requirements, design, and architecture are settled before each piece is committed to. With the core principles now consolidated, proof-of-concept implementation has begun, taking the web configuration surface first (it is the seam that forces the tag/metadata contract to be defined). The web app has been scaffolded (Vite + React + Bootstrap 5, with a swappable theme layer) and the Overview page builds against the pool’s SYS block.

# 2. Product Vision

A smart industrial HMI / mini-PLC / Data-Concentrator (DCU) product. The long-term intent is a flexible platform, not a single fixed-function device: a strong protocol-agnostic core onto which protocols, logic, communication, web, and UI capabilities can be added or replaced over the product’s life without disturbing the rest.

## 2.1 Multipurpose, Configuration-Defined Device

The product is not a PLC, a DCU, or an IoT gateway. It is one platform that becomes any of those depending on configuration. The same hardware and the same firmware image are deployed everywhere; the user selects the application role through the web interface, and from that point the device behaves and presents as that kind of product.

The configuration defines the persona. This is the externally visible product identity — what the customer actually buys — and it is the natural consequence of the flexible architecture described in Section 5: the architecture is the means, multipurpose-by-configuration is the meaning.

This introduces a first-class concept — the Application Profile / Device Role — a configuration layer that sits above the individual seams and governs which plugins/seams are active, how the web UI and HMI present (which pages and tabs are relevant), and the defaults and guardrails for that role.

Two flavours of “multipurpose” are possible and the architecture supports both: pick-a-mode (the unit is one role at a time — appliance-like) and à-la-carte / combinable (independent seams enabled per use, so one box can be PLC and DCU and gateway simultaneously — platform-like). Pick-a-mode is simply a friendly preset on top of the combinable engine, so only the presentation choice remains open.

# 3. Working Approach (how this project is run)

- Requirements first. Implementation is paused until requirements, design, and architecture are clear.

- Sequence: think → consolidate requirements → design → architecture → then implement (implementation is last).

- Founder-led discovery. Requirements are derived by studying existing market products and identifying where this product can be uniquely better.

- Collaborative, incremental. Principles are introduced one at a time, absorbed, and reflected back before any decision is made.

- No premature decisions. Shared context is built up first.

# 4. Target User & Market Strategy

- Beachhead (sell/build first): Industrial automation. The v1 reference experience is built around this vertical.

- Platform stays hybrid underneath: the same core can also serve energy metering (DCU), IIoT/AIoT gateway, and similar roles per deployment.

- Differentiation matters: there must be a clear answer to “why not buy an existing product?” The reference competitor to study and beat is Revolution Pi (RevPi) by Kunbus (an RPi-based industrial hybrid running CODESYS or Linux apps). Identified gaps to exploit: DLMS/COSEM is poorly served in the cheap-edge space, and integrated web-based programming + local HMI is usually two separate tools.

See Section 6 for the full market-differentiation analysis derived from a competitive scan of this category.

# 5. Core Architectural Principle — Flexibility Across Every Layer

The #1 goal: the architecture must let any new protocol, new mechanism, or update be plugged in without disturbing the rest of the system. This must hold at every layer — protocols, communication, web application, and the graphical UI (graphical objects for the PLC).

Discipline (the right way to be flexible): flexibility is achieved by defining stable contracts at well-defined seams, not by making everything dynamic. “Add a new implementation behind a contract without touching anything else” is the operational definition of flexible here. Gratuitous abstraction and over-engineering are avoided.

## 5.1 The Seams (defined extension points)

| Seam | What plugs in | Contract |
| --- | --- | --- |
| South-bound / drivers | Modbus, DLMS, DNP3, OPC-UA, … | driver iface: init / poll / write / status → writes tags |
| Logic / runtime | Ladder, FBD, ST, scripting | reads/writes tags; block iface |
| North-bound / connectors | REST, MQTT, Sparkplug B, cloud, historian | reads tags, publishes; connector iface |
| Web application | per-protocol config pages, dashboards | descriptor/metadata-driven UI |
| HMI graphical objects | PLC screen widgets (gauge, button, trend) | widget binds to a tag by name; widget registry |
| Persistence / alarms / events | retentive, alarm rules, historian | tag-change subscriptions |

## 5.2 The Tag is the Lingua Franca — an On-Device Unified Namespace

The register pool is not just storage — it is the universal bus every seam communicates through. Drivers write tags; logic transforms tags; widgets bind to tags; connectors publish tags. No seam needs to know about any other. This is the central architectural bet and it is already in place (see Section 9).

In current industry terms this internal tag model is a Unified Namespace (UNS) realised on the device itself: a single source of truth that every protocol and application reads from and writes to, replacing point-to-point “integration spaghetti” with a hub-and-spoke data fabric. UNS — typically carried over MQTT with Sparkplug B (birth/death certificates, report-by-exception, ISA-95 topic hierarchy) — is the dominant IIoT data architecture of 2025–2026. The platform should expose a Sparkplug B north-bound connector so the device plugs directly into a customer’s existing UNS, and so its DLMS readings, PLC I/O, and gateway data all live in one coherent tag model.

Required enrichment: tag metadata (name, type, engineering units, scaling, R/W access, quality, timestamp, source) so the web and HMI layers can render any new protocol’s data generically, with no new code. This is the same descriptor-driven idea that schema-driven config frameworks use, applied to tags.

## 5.3 The Web Visual Theme is a Swappable Layer

The web application’s look is itself a seam, in keeping with the flexibility principle: the theme must be changeable at any time without rewriting the application. This is enforced by a three-layer separation in the web codebase.

| Layer | Contains | Theme-coupled? |
| --- | --- | --- |
| 1. Data / logic | API client, state, domain models (tags, profiles, protocols) | No — zero CSS, framework-agnostic |
| 2. Presentational components | cards, tables, forms, the app shell (sidebar/topbar) | Yes — thin, built against the theme |
| 3. Theme tokens | CSS custom properties (colours, fonts, metrics), light/dark | Yes — this is “the theme” |

Two levels of theme change: (1) re-skin within the same framework (colours, brand, density, light/dark) — edit only the Layer-3 token file; runtime-switchable. (2) full template/framework swap — replace Layers 2 and 3 only; the data/logic layer is untouched, so it is a presentation-only job, never a rewrite. Discipline that makes this hold: no business logic in presentational components, and no hardcoded brand colours — always via a token.

v1 stack decision: Vite + React + Bootstrap 5 (visual reference: a BootstrapMade admin dashboard theme), Bootstrap Icons, and ApexCharts for trends; light/dark switchable from day one. Note — licensing: a commercial template license is required before shipping a sold product with the theme embedded in firmware.

# 6. Market Differentiation

Derived from a competitive scan of the small embedded-Linux / RPi industrial-controller category, the software-defined-PLC trend, the IIoT/AIoT gateway segment, and the energy-DCU / DLMS market. The finding: every segment holds one piece of this product, but no incumbent holds the combination — and several are structurally prevented from offering it.

## 6.1 The White Space

| Segment | Who is there | What they leave open |
| --- | --- | --- |
| RPi industrial PLCs | RevPi, UniPi, Industrial Shields, Seeed | No integrated touchscreen + web PLC IDE; no true config-selectable persona; no DLMS |
| Software-defined PLC | Siemens S7-1500V, CODESYS Virtual, Bosch ctrlX, Beckhoff | Enterprise / IT-server priced; “software defines function” but on costly iron, not cheap edge |
| IIoT / AIoT gateways | Advantech, Moxa, Teltonika, Sierra | “Programmable,” not role-redefinable — you write the app; the device role is fixed |
| Edge data brokers | Omron DX1 (Data Flow Controller) | Validates the data-broker half, but: no control logic, no local HMI, no config-selectable persona, no DLMS; mid-tier appliance cost |
| Energy DCU / DLMS | Landis+Gyr, Itron, Genus, Secure | Expensive, single-purpose, vendor-locked appliances; DLMS almost absent from general edge boxes |

Closest single analog — Omron DX1: launched 2024, it runs a browser-based no-code data-flow runtime (SpeeDBee Synapse) over an internal time-series store that drivers feed and northbound connectors drain — conceptually adjacent to this product’s tag pool. It deliberately omits the three things that make this product distinct: real PLC control logic, a local touchscreen HMI, and config-selectable personas (including a DLMS energy DCU). Useful anti-positioning: “DX1 moves your data; FlexiDon iX is the controller, the HMI, and the concentrator — configurable per deployment, at a fraction of the hardware cost.”

The gap is the intersection: cheap edge hardware + config-selectable persona (PLC / DCU / gateway) + native DLMS + on-device tag bus (UNS) + local HMI + browser-based programming. Each incumbent has one of these; none has all.

## 6.2 Defensible Differentiators (ranked)

- Persona-by-configuration, not by re-programming. The incumbents sell a runtime to program; this sells a role to choose. “PLC + gateway + edge” is marketing flexibility for them, not a runtime mode switch. The seam architecture makes it real. This is the headline.

- The tag pool is a built-in Unified Namespace. UNS + MQTT Sparkplug B is the dominant IIoT pattern of 2025–2026 (and the emerging i3X standard). “One internal model every protocol and app reads/writes” is exactly the register pool — a UNS built without naming it. Naming it and adding Sparkplug B north-bound plugs straight into customers’ existing UNS.

- Native DLMS/COSEM at cheap-edge price. Pi-class silicon runs Gurux DLMS comfortably, yet energy DCUs are expensive vendor-locked appliances and general edge boxes skip DLMS entirely. Breaking DCU/HES vendor lock-in — so operators are not captive to one meter vendor — is near-empty space.

- Integrated web PLC programming + on-device touchscreen HMI. Rare to unserved: CODESYS is a great IDE but desktop/licensed with no screen; OpenPLC is web-based but has no display; Seeed has the screen but leans on Node-RED. Combining both in one unit is differentiated.

## 6.3 Positioning Statement

A single low-cost edge device that becomes a PLC, an energy data-concentrator, or an IoT gateway by configuration alone — with a built-in Unified Namespace, native DLMS, local touchscreen HMI, and browser-based programming. Three locked products from the incumbents, collapsed into one open, config-defined SKU.

The pain it kills: integration spaghetti (a large share of OT data trapped in silos), three separate procurement lines, and vendor lock-in — none of which incumbents can solve without cannibalising their own business model.

## 6.4 Honest Caveats (where not to over-claim)

RPi-class hardware plus this stack targets the cheap, flexible, data-centric edge. The product should not, yet, claim parity with safety/heavy-duty incumbents on:

- Functional-safety certification (SIL / IEC 61508).

- Hard real-time / motion control.

- Redundancy / failover.

- 24/7 industrial-grade support and long-term lifecycle guarantees.

## 6.5 Vocabulary to Align With

Software-defined automation · IT/OT convergence · Unified Namespace (UNS) · MQTT Sparkplug B · i3X · Industrial DevOps · report-by-exception.

# 7. Hardware Principle

- Hardware does not constrain the design. Design for the target product; size the hardware (RAM, storage, cores) to the architecture — not the reverse.

- The Raspberry Pi is a proof-of-concept only, implementing a minimal slice (one concrete plugin per seam) to prove the contracts work.

- Final product hardware will be specified as needed, with no compromise.

- Consequence: choose the most modular / most isolated architecture for flexibility and maintainability; runtime frugality is a quality applied within the design, not a constraint that shapes it.

# 8. Open Decisions (not yet settled)

- Tag / metadata model — the exact schema describing a tag beyond its raw value. Foundation everything else binds to.

- Firmware extension & isolation model — how drivers/logic/connectors are delivered and contained. Options: separate processes over the shared pool (fault-isolated) / dynamic .so plugins (dlopen) / compile-time modules / hybrid. Leaning toward the most modular option now that hardware is not a constraint.

- Multipurpose presentation model — pick-a-mode vs à-la-carte/combinable as the user-facing experience (the engine supports both; only the presentation is open). See Section 2.1.

- v1 differentiation specifics vs RevPi and other incumbents — now informed by Section 6.

Evaluated and deferred: OpenWrt + LuCI as the web/admin framework. LuCI is an OpenWrt-native device-admin tool; adopting it would fragment the single-config-surface decision and contributes nothing to the PLC-programming / tag-dashboard differentiation. Revisit only if network-device administration becomes a headline feature — in which case the real decision is basing the product on OpenWrt, not bolting LuCI onto Yocto.

# 9. What Has Been Built (PoC Status Snapshot)

| Component | Status |
| --- | --- |
| libmdcu-pool 0.1 | 50,000 × 16-bit shared-memory register pool at /dev/shm/mdcu_pool; typed API (bit, u8/i8, u16/i16, u32/i32, u64/i64, f32, f64) + bulk block ops; ARMv6-safe atomics + seqlock; canonical address map. Realizes the “tag is the lingua franca” / on-device UNS bet. |
| LVGL HMI (miniplc-hmi r11) | Touchscreen UI; now read-only for protocol data, with the web UI as the sole configuration surface. All protocol CONFIG modals removed. Settings tab (Network + Theme) remains interactive. Writes system tags (CPU temp, uptime, epoch, heartbeat); mirrors Modbus poll results into the pool. |
| Modbus TCP master | Enabled, polling, mirroring to the pool via a libmodbus bridge that isolates symbol clashes. |
| PLC firmware | civetweb REST API, ladder VM, GPIO (foundation present; protocol pollers + /api/regs still to come). |
| Web UI | React/Vite SPA served by nginx; protocol config pages exist as components, to be wired to live data. |
| Verification | Confirmed on device with the mdcu-peek debug tool (pool header valid, system tags ticking, Modbus values mirrored). |

# 10. Immediate Next Steps (once requirements are settled)

- Settle the tag/metadata model (Open Decision #1) — design it as the on-device UNS schema, metadata-rich enough to drive generic web/HMI rendering and a Sparkplug B mapping.

- Settle the extension & isolation model (Open Decision #2).

- Decide the multipurpose presentation model (Open Decision #3).

- Then resume implementation against the agreed contracts, starting with the industrial-automation beachhead and one concrete plugin per seam.

End of document — FlexiDon iX Requirements & Project Consolidation, v0.4